Additional information

- The Github repository includes raw data of structural characterization (XRD, XRR, Phi-scan, AFM), interface characterization (STM & TOF-SIMS), magnetic & transport characterization of pristine Nb film and Mn3Pt/Nb bilayers and I-V sweeps.

- Data measurement: I-V sweeps are measured in Quantum Design (QD) PPMS using MultiVu software. Raw I-V data are uploaded in the repository.

- All data are analyzed and plotted using OriginLab. Differential resistance of I-V curves are plotted using Matlab codes uploaded in repository.

- Estimation of diode efficiency: Import the raw I–V data in Originlab  separate positive and negative current branches  extract Ic+ and Ic−  calculate η  determine σIc = ΔI/2  propagate the uncertainties to obtain ση.

- Independent repeated sweeps are processed separately.

- The raw I–V curve is inspected to identify the transition from the superconducting state to the resistive state, where a finite voltage develops. The critical current is determined consistently using the same criterion for all datasets. In the present analysis, the abrupt onset of the resistive state/switching in the I–V characteristic is used to identify the critical current.

- For the positive-current branch, the current at which the device switches from the superconducting to the resistive state is designated as Ic+. For the negative-current branch, the corresponding switching current is designated as Ic−. Because Ic− is negative by convention, its magnitude |Ic−| is used when calculating the diode efficiency.